# Filter design for GEANT fibre links in the C-TFN

## Context

GEANT is building optical fibre links for the C-TFN that will interconnect European NMIs.  These optical links require bi-directional EDFA amplifiers roughly every 70km.  Considering that the fibre is expected to be in place for up to 10 years and mid-life upgrading of equipment will be difficult, it is important to ensure that the filter design is future-proofed; we need filters to support all current and future services envisaged for the C-TFN.

The proposed DWDM filters will allow amplifiers, ELSTAB equipment and White Rabbit equipment can be added at amplifier site as needed.

## Spectrum

Dense Wavelength Division Multiplexing (DWDM) is divided up over several optical bands, the bands S, C and L are summarised below.

| Band | Name | Wavelength Range (nm) |
| --- | --- | --- |
| S-band | Short Wavelength Band | 1460–1530 nm |
| C-band | Conventional Band | 1530–1565 nm |
| L-band | Long Wavelength Band | 1565–1625 nm |

The C-band is the most widely used in long-haul DWDM because it has the lowest attenuation and is compatible with EDFA (Erbium-Doped Fiber Amplifiers). The L-band allows network providers to expand capacity and is also EDFA-compatible.  The S-band can be used in cases where additional capacity is needed but the S-band is not EDFA compatible.

The original C-band (1530nm-1565nm) has been expanded twice since its first introduction to provide more channels and increase network capacity by extending it into the S and L bands: Extended C-band (1530nm–1570 nm) and the Super C-band (1525–1575 nm).  These bands are depicted in appendix A.

## Design

There are two vendors of bi-directional optical amplifier commonly used by the NMI community which will form the basis of this design proposal, these are:

- Keopsys  https://www.keopsys.com/portfolio/bi-directional-fiber-amplifier/

- PIK Time Systems: https://piktime.com/offer/oba-3/

Other amplifiers are also available, but are not considered in this analysis.

It is not known which of these amplifiers will be chosen in GEANT’s public procurement process, for this reason two designs are proposed each using one of the two amplifier types.   The design of the amplifiers used in the C-TFN pathfinder are shown in Appendix B.  The design of the OBA-3 amplifier is shown in Appendix C.

Option 1, OBA-3:

Option 1 is designed around the use of the OBA-3 bi-directional optical amplifiers from PIK Time Systems.  This amplifier has an in-built filter and an express port, for details see Appendix D.  For this design I have assumed that the OBA-3 would be delivered with the 8skip1 filter centred on Ch44.  It is assumed that the performance of the 8skip1 filter conforms to this specification:
https://www.iridian.ca/product/dwdm-8s1-1559-39/

In this design the output of the OBA-3 amplifier express port is forwarded to an external filter housed in an external rack mount chassis

Option 2, Keopsys

Option 2 is designed around the use of the Keopsys bi-directional optical amplifier.  This amplifier does not have an in-built filter.

In this design the filters for frequency, WR, Elstab and optical supervisory channels is housed in an external box.  The frequency plan remains the same.

The external C-band DWDM filter is connected to the Keopsys as shown in the diagram below:

## Spectrum allocation for C-TFN filters

The diagram and table below show a possible spectrum allocation plan for the C-TFN.

- Channels 49-69 are available for White Rabbit

- Channels 41-47 are available for frequency (and ELSTAB if needed)

- Channels 33-39 are for other uses

- CWDM channels 1571nm and 1591nm are for CWDM OSCs

## Appendix A: Pathfinder filter:

The diagram below shows the filter spectrum as used in the C-TFN pathfinder link

## Appendix B: OBA3 from PIK Time Systems

The two diagrams below show the design of the OBA3 amplifier.