Filter design for GEANT fibre links in the C-TFN

## Overview

This document outlines the proposed design for optical filters in the GEANT links within the C-TFN network. These filters are designed to partition the optical spectrum, ensuring the fibre can support all planned services while also being adaptable to accommodate future services.

Contents

Overview	1

Context	2

Spectrum	2

Fibre	2

Amplifier Hardware	2

Spectrum allocation for C-TFN filters	3

Design	4

Appendix A: ITU-T spectrum	6

Appendix B: Pathfinder filter:	7

Appendix C: OBA3 from PIK Time Systems	8

Appendix D: CESNET filter design	9

## Context

GEANT is building optical fibre links for the C-TFN that will interconnect European NMIs.  These optical links require bi-directional EDFA amplifiers roughly every 70km.  Considering that the fibre is expected to be in place for up to 10 years and mid-life upgrading of equipment will be difficult, it is important to ensure that the filter design is future-proofed; we need a design for filters that will support all current and future services envisaged for the C-TFN.

The proposed DWDM filters should support bi-directional amplifiers for frequency transmission and ELSTAB equipment.  They should also allow White Rabbit equipment to be added at each amplifier site as needed.

### Spectrum

Dense Wavelength Division Multiplexing (DWDM) is divided up over several optical bands, the bands S, C and L are summarised below.

| Band | Name | Wavelength Range (nm) |
| --- | --- | --- |
| S-band | Short Wavelength Band | 1460–1530 nm |
| C-band | Conventional Band | 1530–1565 nm |
| L-band | Long Wavelength Band | 1565–1625 nm |

The C-band is the most widely used in long-haul DWDM because it has the lowest attenuation and is compatible with EDFA (Erbium-Doped Fiber Amplifiers). The L-band allows network providers to expand capacity and is also EDFA-compatible.  The S-band can be used in cases where additional capacity is needed but the S-band is not EDFA compatible.

The original C-band (1530nm-1565nm) has been expanded twice since its first introduction to provide more channels and increase network capacity by extending it into the S and L bands: Extended C-band (1530nm–1570 nm) and the Super C-band (1525–1575 nm).  These bands are depicted in appendix A.

### Fibre

GEANT will acquire a pair of fibres for use in the C-TFN. The filters described herein will be installed on both fibres, irrespective of whether amplifiers are initially deployed on one or both.

### Amplifier Hardware

There are two vendors of bi-directional optical amplifier commonly used by the NMI community which will form the basis of this design proposal, these are:

- Keopsys  https://www.keopsys.com/portfolio/bi-directional-fiber-amplifier/

- PIK Time Systems: https://piktime.com/offer/oba-3/

Other amplifiers are also available, but are not considered in this analysis.

It is not known which of these amplifiers will be chosen in GEANT’s public procurement process, for this reason two designs are proposed each using one of the two amplifier types.

## Spectrum allocation for C-TFN filters

The diagram and table below show the proposed spectrum allocation plan for the C-TFN.

- Channels 49-59 are available for White Rabbit

- Channels 41-47 are available for frequency (and ELSTAB if needed)

- Channels 33-39 are for other uses

- OSC channels have two options

  - Option 1: CWDM channels 1570nm and 1590nm

  - Option 2: CWDM channels 1490nm and 1510nm

## Design

The design of the amplifiers used in the C-TFN pathfinder are shown in Appendix B.  The details of the OBA-3 amplifier is shown in Appendix C.

Option 1, OBA-3:

Option 1 is designed around the use of the OBA-3 bi-directional optical amplifiers from PIK Time Systems.  This amplifier has an in-built filter and an express port, for details see Appendix D.  For this design I have assumed that the OBA-3 would be delivered with the 8skip1 filter centred on Ch44.  It is assumed that the performance of the 8skip1 filter conforms to this specification:
https://www.iridian.ca/product/dwdm-8s1-1559-39/

In this design the output of the OBA-3 amplifier express port is forwarded to an external filter housed in an external rack mount chassis

Option 2, Keopsys

Option 2 is designed around the use of the Keopsys bi-directional optical amplifier.  This amplifier does not have an in-built filter.

In this design the filters for frequency, WR, Elstab and optical supervisory channels is housed in an external box.  The frequency plan remains the same.

The external C-band DWDM filter is connected to the Keopsys as shown in the diagram below:

## Appendix A: ITU-T spectrum

The following diagram shows the ITU-T CWDM and DWDM bands and channels

## Appendix B: Pathfinder filter:

The diagram below shows the filter spectrum as used in the C-TFN pathfinder link

## Appendix C: OBA3 from PIK Time Systems

The two diagrams below show the design of the OBA3 amplifier.

## Appendix D: CESNET filter design

In the CESNET network, pre-filters are installed on most optical fibers on the line side of the DWDM equipment. These pre-filters enable the coexistence of a commercial DWDM system with non-data services such as the transmission of accurate time and coherent frequency and also allows for future expansion.   External filters:

- Bi-directional operation for time and frequency:  A 8skip0 filter on channels 46 to 39 (1540.56 - 1546.12 nm)

- Future use: A 4skip0 filter is deployed on ITU channels 9 to 6 (1570.42 - 1572.89 nm)

Filters in the Ribbon DWDM system:

- White Rabbit:  DWDM ITU channels 30-35 (1553.33 - 1549.32 nm) are reserved on the Ribbon DWDM system for WR.

- OSC channels : The Ribbon DWDM system uses the standard at 1510 nm channel (1510 and 1590 nm in the case of single fiber)

- OTDR channel: The Ribbon DWDM system uses 1610 nm channel.